Problem I: Attitude Determination

Given: Image of stars; Inertial-Frame unit vectors

To Find: Body-Frame unit vectors; Rotation matrix using various methods; Error;

Part A) Extracting Body Frame unit vectors

Figure 1. Raw image

The first step was to make sure that Matlab was properly able to open the .png file. The following image shows that Matlab succefully opens the image, as shown in Figure 1.

The next step was to extract all the stars in the image, regardless of its brightness. The following image shows just that.

Figure 2. Identified stars

The next step is to find the centroid of each of these stars in the image. Specifically, the x & y pixel coordinates, whose origin is at the top left corner, were found. The following is that data:

Figure 3. Image with the stars of interest labeled

However, we do not care about the coordinates of every star. We only want to analyze the ones labeled in this image:

Using Matlab’s built-in feature, the coordinates of these stars were approximately found as follows:

Next, these approximate coordinates were compared to the centroid found earlier to get more exact coordinates. This was done using the following equation:

Figure 4. Centroid locations plotted

This yields the following “exact” coordinates of the centroid of each star. These coordinates were overlayed onto the image to verify that we have identified the correct stars and compared to Figure 3.

Table 1. Exact coordinates of the stars of interest

| Star # | x | y |
| --- | --- | --- |
| 1 | 86.3143 | 19.8000 |
| 2 | 150.1887 | 13.5660 |
| 3 | 9.7843 | 114.3137 |
| 4 | 59.5000 | 93.5000 |
| 5 | 149.5294 | 101.4314 |

The next step is to convert these pixel coordinates to actual, physical coordinates in the body frame. This is done using the following equation:

where P is the physical coordinate, W is the width of the image, and FOV is the field of view of the camera (4 degrees in this case). These coordinates were then normalized to become unit vectors. The table below summarizes this data.

Table 2. Body-Frame Coordinates of the stars of interest

| Star # | b1 | b2 | b3 |
| --- | --- | --- | --- |
| 1 | -0.1587 | -0.9298 | 0.332 |
| 2 | 0.4827 | -0.8313 | 0.2754 |
| 3 | -0.9424 | 0.1495 | 0.2991 |
| 4 | -0.8096 | -0.1299 | 0.5724 |
| 5 | 0.8654 | 0.025 | 0.5004 |

Part B) Inertial Frame Unit Vectors

The inertial frame unit vectors of these five stars are known and were hard-coded as follows:

Table 3. Inertial-Frame Coordinates of the stars of interest

| Star # | i1 | i2 | i3 |
| --- | --- | --- | --- |
| 1 | -0.9211 | -0.3426 | 0.1851 |
| 2 | -0.4770 | -0.7120 | 0.5154 |
| 3 | -0.4966 | 0.8168 | -0.2937 |
| 4 | -0.7362 | 0.6766 | 0.0104 |
| 5 | 0.3047 | -0.2994 | 0.9042 |

Part C) Attitude Estimation using TRIAD

The TRIAD method only uses two measurements to estimate the attitude of the spacecraft. For this, I will choose stars 2 and 3 because they are further apart so precision errors will hopefully not be much of an issue.

Doing the TRIAD method, the DCM that relates the body frame to the inertial frame is as:

CBN0.5348-0.62360.57010.76650.6420-0.0167-0.35560.44600.8214

Next, the error was calculated using:

Where b is the body-frame vectors and r is the inertial frame vectors. Taking the magnitude of this error vector yields an error of 0.0158.

Part D) Attitude Estimation using OLAE

The OLAE method uses all the information to estimate the attitude of the spacecraft. From this, the resulting DCM is

CBN0.5321-0.62500.57120.76730.6411-0.0134-0.35780.44540.8207

and the error is 0.0140.

Part E) Attitude Estimation using Davenport q-method

The Davenport q-method also uses multiple measurements, but is also better adapted for noise in the measurements. Using this metho, the resulting DCM is:

CBN0.5321-0.62480.57140.76720.6413-0.0133-0.35820.44550.8205

and the error is 0.0140.

When comparing the error of the three methods, the TRIAD method has the worst (largest) error. However, the OLAE method has the same error as the Daveport q-method. This is expected because the latter two methods benefit from taking additional measurements while the TRIAD only uses two measurements. Evidently, utilizing more data in the calculations leads to more accurate results.

Problem I (Code)

% AERSP 450 HW5 - Question I

% Made by Nicholas Luis (PSU ID: 930841391)

% Remove semi-colons to see the output

%% Parts A & B - Basic Data

clc; clear; close all;

% Step 1

grayImg = imread('StarField.png');

imshow(grayImg);

% Step 2

binaryImg = imbinarize(grayImg, 'adaptive'); %Adaptive thresholding

imshow(binaryImg)

% Step 3

stats = regionprops(binaryImg, 'Centroid');

centroids = cat(1, stats.Centroid); %Extract x and y pixel coordinates

% Step 4 : Comparing extracted coords to manually-obtained coords

Star1 = [86,19];

Star2 = [150,14];

Star3 = [10,114];

Star4 = [60,93];

Star5 = [150,101];

starCoords = [Star1; Star2; Star3; Star4; Star5];

Cents = NaN(length(starCoords),2);

for i=1:size(starCoords)

[M,I] = min(((centroids(:,1)-starCoords(i,1)).^2 + (centroids(:,2)-starCoords(i,2)).^2).^(0.5));

Cents(i,:) = centroids(I,:); % Saves new centroid coordinates

end

clear i;

% Step 5: Verifying coordinates

imshow("StarField.png")

hold on

plot(centroids(:,1),centroids(:,2),'.r')

hold on

plot(Cents(:,1),Cents(:,2),'og')

% Step 6: Converting pixel coords into spatial

[L,W] = size(binaryImg); % Gets image dimensions

P = (Cents - W/2) * tan((4*pi/180)/2);

P = cat(2,P,ones(5,1));

for j = 1:length(P)

P(j,:) = P(j,:)./norm(P(j,:)); % Normalize vectors to get unit vector

end

clear j;

% (Part B)

StarI1 = [-0.921069884293268 -0.342599924017704 0.185082577005643];

StarI2= [-0.476980639452282 -0.711962047538100 0.515363476056510];

StarI3= [-0.496592767571065 0.816782636490539 -0.293703503424244];

StarI4= [-0.736236774114155 0.676644212846351 0.010393347079969];

StarI5 = [ 0.304668730748568 -0.299446239108458 0.904161995655567];

starCoordsInertial = [StarI1; StarI2; StarI3; StarI4; StarI5];

close all;

%% Part C (TRIAD Method)

% Step 1: Computing DCM

b1 = P(2,:)';

b2 = P(3,:)';

b3 = cross(b1,b2) / norm(cross(b1,b2));

r1 = starCoordsInertial(2,:)';

r2 = starCoordsInertial(3,:)';

r3 = cross(r1,r2) / norm(cross(r1,r2));

B = [b1, b3, cross(b1,b3)];

R = [r1, r3, cross(r1,r3)];

C_BN_triad = B*R';

% Step 2

errorMat_Triad = NaN(3,length(starCoordsInertial));

for k = 1:length(starCoordsInertial)

errorMat_Triad(:,k) = P(k,:)' - (C_BN_triad*starCoordsInertial(k,:)');

end

clear k;

errorTriad = norm([errorMat_Triad(1,:), errorMat_Triad(2,:), errorMat_Triad(3,:)]);

%% Part D (OLAE Method)

S_matrix = NaN(3*length(P), 3);

D_vector = NaN(length(P), 3);

for n = 1:length(P)

S = P(n,:)' + starCoordsInertial(n,:)';

D_vector(n,:) = P(n,:)' - starCoordsInertial(n,:)';

cntr = 1 + 3*(n-1);

S_matrix(cntr:3*n, :) = skewSymmetric(S);

end

clear n; clear cntr;

D_long = reshape(D_vector',[],1); % Long vector

q = pinv(S_matrix)*D_long;

% DCM relating body to inertial frame using OLAE method

C_BN_olae = ((eye(3) + skewSymmetric(q)) / (eye(3) - skewSymmetric(q)))';

% Calculating error

errorMat_OLAE = NaN(3,length(starCoordsInertial));

for k = 1:length(starCoordsInertial)

errorMat_OLAE(:,k) = P(k,:)' - (C_BN_olae*starCoordsInertial(k,:)');

end

clear k;

errorOlae = norm([errorMat_OLAE(1,:), errorMat_OLAE(2,:), errorMat_OLAE(3,:)]);

%% Part E (Davenport q-method)

B_mat = zeros(3,3);

for i = 1:length(P)

B_mat = B_mat + ( P(i,:)' * starCoordsInertial(i, :) );

end

sigma = trace(B_mat);

S = B_mat + B_mat';

Z = [B_mat(2,3) - B_mat(3,2);

B_mat(3,1) - B_mat(1,3);

B_mat(1,2) - B_mat(2,1); ];

% K matrix

K = [sigma, Z';

Z(1), S(1,:) - [sigma, 0, 0];

Z(2), S(2,:) - [0, sigma, 0];

Z(3), S(3,:) - [0, 0, sigma]; ];

% Eigen-stuff

[eVec, eVal] = eig(K);

[~, I] = max(diag(eVal));

q = eVec(:,I);

% Converting quaternion to DCM

C_BN_dave = [ q(1)^2 + q(2)^2 - q(3)^2 - q(4)^2, 2*(q(2)*q(3) + q(1)*q(4)), 2*(q(2)*q(4) - q(1)*q(3)) ;

2*(q(2)*q(3) - q(1)*q(4)), q(1)^2 - q(2)^2 + q(3)^2 - q(4)^2, 2*(q(3)*q(4) + q(1)*q(2)) ;

2*(q(2)*q(4) + q(1)*q(3)), 2*(q(3)*q(4) - q(1)*q(2)), q(1)^2 - q(2)^2 - q(3)^2 + q(4)^2 ;]

% Calculating error

errorMat_dave = NaN(3,length(starCoordsInertial));

for k = 1:length(starCoordsInertial)

errorMat_dave(:,k) = P(k,:)' - (C_BN_dave*starCoordsInertial(k,:)');

end

clear k;

errorDave = norm([errorMat_dave(1,:), errorMat_dave(2,:), errorMat_dave(3,:)])

%% Functions

function matrixTilda = skewSymmetric(vec)

% This function inputs a vector and returns a skew symmetrix matrix

matrixTilda = [0, -vec(3), vec(2);

vec(3), 0, -vec(1);

-vec(2), vec(1), 0;];

end

Problem II: Attitude Dynamics: Analytical

Problem III: Attitude Dynamics: Numerical

Part A) Data handling and preliminaries

- Given: Measurements of an iPhone
Find: 	Principal moments of inertia assuming rectangular, cuboid shape

Figure 5. Moment of inertia for a rectangular prism.

Source: "Mass and Area Moments of Inertia in SOLIDWORKS", wiki.cadcam.com.my/knowledgebase/mass-and-area-moments-of-inertia-in-solidworks/

The equations for principal moments of inertia is given by the following:

Figure 6. Provided coordinate system of the phone

For consistency, I have set the coordinate x-y-z coordinate system in Figure 6 to align with the A-B-C system in Figure 5. That means:

- a = 71.5 mm

- b = 7.4 mm

- c = 146.7 mm

Given m = 189g, the following are the principal moments of inertia:

Ix=33.9816*104 [g∙mm2]
Iy=8.1380  *104 [g∙mm2]
Iz=41.9471*104 [g∙mm2]

- A simple comparison of the numbers tells us: 
The y-axis is the minimum moment of inertia axis
The x-axis is the intermediate
& the z-axis is the maximum

Figure 7. Individual angular velocity components plotted vs time. The red dashed lines indicate the region where the phone was in the air.

Plotting the given angular data yields

Times in which the phone was in the air was identified because, when zoomed in, the angular velocities were sinusoidal. Specifically, the phone was in the air between 5.10 and 5.98 seconds for experiment 1.

Figure 8. Individual angular velocity components plotted vs time

Figure 9. Individual angular velocity components plotted vs time

Repeating this process for experiments 2 and 3 yield the following plots:

For experiment 2, the phone was in the air between and 3.19 and 4.12 seconds.
For experiment 3, the phone was in the air between and 4.63 and 5.42 seconds

Part B) The ‘Fun’ Part

- Experiment 1 represents a rotation about the minor axis of inertia, Iy. This is because based on Figure 7, the angular velocity about the y-axis, ωy, was roughly constant. Not only that, but the angular velocity also had a very large magnitude compared to the other subplots. 

Using similar logic: Experiment 2 represents a rotation about the major axis of inertia, Iz. This is because based on Figure 8, the angular velocity about the z-axis, ωz, was roughly constant and very large magnitude.

Experiment 3 is difficult to determine, but it most likely represents a rotation about the intermediate axis, Ix. Based on Figure 9, the angular momentum about the x-axis, ωx, had a large magnitude. ωy also had a large magnitude, but it was not a sinusoidal shape.

- Analyzing the energy and angular momentum of the system.

The total energy of the system at any moment in time is given by:

KE = 12(Ixωx2+Iyωy2+ Izωz2)

Figure 10. Note that the airtime is when the energy suddenly spikes.

The following is a plot of the total energy of the system as a function of time:

The angular momentum of each component is given by:

Hi=Iiωi

Figure 11. Note that the airtime is when the energy suddenly spikes.

The total angular momentum of the system is given by simply taking the magnitude of Hi. The following is a plot of the total angular momentum of the system vs time:

The phone is in the air during the spike that lasts around a second. Even while it is in the air, the energy is not constant. This is especially apparent in Experiment 2, which has a noticeable decrease while it is in the air. Similarly, the angular momentum plots indicate that the angular momentum is not necessarily constant. Thus, neither energy nor angular momentum are conserved. In real life, drag from the air causes a torque that slows down the rotation, which appears as a decrease both the angular momentum and energy.

- Propagating Euler’s Equations of Motion at the beginning of the airtime & comparing it to the experimental results.

The propagation was done using ODE45 in Matlab and the following equations of motion:

Euler’s Equations of Motion. Source: “Euler's equations (rigid body dynamics)”, accessed via Wikipedia.org

Note that these equations are only valid for Torque-Free rotation, which was not the case seen in part 2.

The following are the plots of the propagated data angular velocities compared to the experimental data.

Figure 12 - Minor Axis

Figure 13 – Major Axis

Figure 14 – Intermediate Axis

When comparing the analytical to experimental results, it is clear that the trends are accurate. However, there is some discrepancies. For example, in Figure 12 (experiment 1), there is a large error between the analytical and the experimental. Figure 13 (experiment 2) shows the opposite of this; The analytical solution starts off very close but drifts over time. The same can be said about Figure 14 (experiment 3). It appears that rotation about the minor axis is the least predictable, but that is to be expected because it is inherently an unstable system; With perturbations (in this case drag), objects will tend to rotate around their major axis.

- Using the following equation:

Rotation around the minor axis is unstable.

Problem III (Code)

% AERSP 450 HW5 - Question III

% Made by Nicholas Luis (PSU ID: 930841391)

% Remove semi-colons to see the output

clear; clc; close all;

%% Part A

% Known properties

h = 146.7; % mm

w = 71.5; % mm

t = 7.4; % mm

m = 189; % g

%Calculate moments of inertia

Ix = (1/12) * m * (h^2 + t^2);

Iy = (1/12) * m * (w^2 + t^2);

Iz = (1/12) * m * (h^2 + w^2);

%load('Experiment1.mat')

% load('Experiment2.mat')

load('Experiment3.mat')

wx = AngularVelocity.X;

wy = AngularVelocity.Y;

wz = AngularVelocity.Z;

T = AngularVelocity.Timestamp;

timeVec = datetime(T, 'InputFormat', 'dd-MMM-yyyy HH:mm:ss.SSS');

t = seconds(timeVec - timeVec(1));

figure(1);

hold on;

subplot(3,1,1);

plot(t, wx);

%     title('Experiment 1 Data');

%     title('Experiment 2 Data');

title('Experiment 3 Data');

ylabel('\omega_x (rad/s)');

%xline([5.102, 5.975], '--r'); % For experiment 1

%     xline([3.18599, 4.11899], '--r'); % For experiment 2

xline([4.6349, 5.42], '--r'); % For experiment 3

subplot(3,1,2);

plot(t, wy);

ylabel('\omega_y (rad/s)');

%     xline([5.102, 5.975], '--r'); % For experiment 1

%     xline([3.18599, 4.11899], '--r'); % For experiment 2

xline([4.6349, 5.42], '--r'); % For experiment 3

subplot(3,1,3);

plot(t, wz);

xlabel('Time (s)');

ylabel('\omega_z (rad/s)');

%     xline([5.102, 5.975], '--r'); % For experiment 1

%     xline([3.18599, 4.11899], '--r'); % For experiment 2

xline([4.6349, 5.42], '--r'); % For experiment 3

hold off;

%% Part B

clear; clc; close all;

% VALUES CONVERTED TO METERS AND KILOGRAMS TO GET JOULES DURING ENERGY CALCULATIONS

% Known properties

h = 146.7/1000; % m

w = 71.5/1000; % m

t = 7.4/1000; % m

m = 189/1000; % kg

%Calculate moments of inertia

Ix = (1/12) * m * (h^2 + t^2);

Iy = (1/12) * m * (w^2 + t^2);

Iz = (1/12) * m * (h^2 + w^2);

% Loading Data

load('Experiment1.mat')

wx1 = AngularVelocity.X;

wy1 = AngularVelocity.Y;

wz1 = AngularVelocity.Z;

T1 = AngularVelocity.Timestamp;

timeVec1 = datetime(T1, 'InputFormat', 'dd-MMM-yyyy HH:mm:ss.SSS');

t1 = seconds(timeVec1 - timeVec1(1));

load('Experiment2.mat')

wx2 = AngularVelocity.X;

wy2 = AngularVelocity.Y;

wz2 = AngularVelocity.Z;

T2 = AngularVelocity.Timestamp;

timeVec2 = datetime(T2, 'InputFormat', 'dd-MMM-yyyy HH:mm:ss.SSS');

t2 = seconds(timeVec2 - timeVec2(1));

load('Experiment3.mat')

wx3 = AngularVelocity.X;

wy3 = AngularVelocity.Y;

wz3 = AngularVelocity.Z;

T3 = AngularVelocity.Timestamp;

timeVec3 = datetime(T3, 'InputFormat', 'dd-MMM-yyyy HH:mm:ss.SSS');

t3 = seconds(timeVec3 - timeVec3(1));

% Processing Data

KE1_hist = NaN(length(t1),1);

W1_hist = NaN(length(t1),1);

for i = 1:length(t1)

KE1_hist(i) = (Ix*wx1(i)^2 + Iy*wy1(i)^2 + Iz*wz1(i)^2);

W1_hist(i) = norm([Ix*wx1(i), Iy*wy1(i), Iz*wz1(i)]);

end

KE2_hist = NaN(length(t2),1);

W2_hist = NaN(length(t2),1);

for i = 1:length(t2)

KE2_hist(i) = (Ix*wx2(i)^2 + Iy*wy2(i)^2 + Iz*wz2(i)^2);

W2_hist(i) = norm([Ix*wx2(i), Iy*wy2(i), Iz*wz2(i)]);

end

KE3_hist = NaN(length(t3),1);

W3_hist = NaN(length(t3),1);

for i = 1:length(t3)

KE3_hist(i) = (Ix*wx3(i)^2 + Iy*wy3(i)^2 + Iz*wz3(i)^2);

W3_hist(i) = norm([Ix*wx3(i), Iy*wy3(i), Iz*wz3(i)]);

end

% Plotting

figure(2)

hold on

plot(t1, KE1_hist, LineWidth=2)

plot(t2, KE2_hist, LineWidth=2)

plot(t3, KE3_hist, LineWidth=2)

legend('Experiment 1 ', 'Experiment 2', 'Experiment 3')

title('Total Energy')

xlabel("Time (s)")

ylabel("Energy (J)")

hold off

exportgraphics(gca,"HW5_Problem3_Energy.jpg");

figure(3)

hold on

plot(t1, W1_hist, LineWidth=2)

plot(t2, W2_hist, LineWidth=2)

plot(t3, W3_hist, LineWidth=2)

legend('Experiment 1 ', 'Experiment 2', 'Experiment 3')

title('Total Angular Momentum')

xlabel("Time (s)")

ylabel("Angular Momentum (kg*m^2/s)")

hold off

exportgraphics(gca,"HW5_Problem3_Momentum.jpg");

% The following are the regions of times that the phone is in the air (from part A)

times = [[5.102, 5.975];

[3.18599, 4.11899];

[4.6349, 5.42]];

% ---------Experiment 1 Progagation---------

t_range = times(1,:);

% Initial conditions

index = min(find(t1 >= 5.102));

wx1_0 = wx1(index);

wy1_0 = wy1(index);

wz1_0 = wz1(index);

w1_0 = [wx1_0, wy1_0, wz1_0]; % Vector of initial conditions for experiment 1

EOMs = @(t, omega) [ ((Iy - Iz) / Ix) * omega(2) * omega(3);

((Iz - Ix) / Iy) * omega(3) * omega(1);

((Ix - Iy) / Iz) * omega(1) * omega(2)];

% Progation using ode45

[t1_prop, omega1_prop] = ode45(EOMs, t_range, w1_0);

% Plotting

figure(4);

subplot(3,1,1);

plot(t1_prop, omega1_prop(:,1), '--r');

hold on;

title('Experiment 1 Propagated Data');

plot(t1(index:index+length(t1_prop),:), wx1(index:index+length(t1_prop),:), 'b');

hold off;

legend('Analytical', 'Experimental')

ylabel('\omega_x (rad/s)');

subplot(3,1,2);

plot(t1_prop, omega1_prop(:,2), '--r');

hold on;

plot(t1(index:index+length(t1_prop),:), wy1(index:index+length(t1_prop),:), 'b');

hold off;

legend('Analytical', 'Experimental')

ylabel('\omega_x (rad/s)');

subplot(3,1,3);

plot(t1_prop, omega1_prop(:,3), '--r');

hold on;

plot(t1(index:index+length(t1_prop),:), wz1(index:index+length(t1_prop),:), 'b');

hold off;

legend('Analytical', 'Experimental')

ylabel('\omega_x (rad/s)');

xlabel('Time (s)');

% ---------Experiment 2 Progagation---------

t_range = times(2,:);

% Initial conditions

index = min(find(t1 >= 3.18599));

wx2_0 = wx2(index);

wy2_0 = wy2(index);

wz2_0 = wz2(index);

w2_0 = [wx2_0, wy2_0, wz2_0]; % Vector of initial conditions for experiment 1

EOMs = @(t, omega) [ ((Iy - Iz) / Ix) * omega(2) * omega(3);

((Iz - Ix) / Iy) * omega(3) * omega(1);

((Ix - Iy) / Iz) * omega(1) * omega(2)];

% Progation using ode45

[t2_prop, omega2_prop] = ode45(EOMs, t_range, w2_0);

% Plotting

figure(5);

subplot(3,1,1);

plot(t2_prop, omega2_prop(:,1), '--r');

hold on;

title('Experiment 2 Propagated Data');

plot(t2(index:index+length(t1_prop),:), wx2(index:index+length(t1_prop),:), 'b');

hold off;

legend('Analytical', 'Experimental')

ylabel('\omega_x (rad/s)');

subplot(3,1,2);

plot(t2_prop, omega2_prop(:,2), '--r');

hold on;

plot(t2(index:index+length(t1_prop),:), wy2(index:index+length(t1_prop),:), 'b');

hold off;

legend('Analytical', 'Experimental')

ylabel('\omega_x (rad/s)');

subplot(3,1,3);

plot(t2_prop, omega2_prop(:,3), '--r');

hold on;

plot(t2(index:index+length(t1_prop),:), wz2(index:index+length(t1_prop),:), 'b');

hold off;

legend('Analytical', 'Experimental')

ylabel('\omega_x (rad/s)');

xlabel('Time (s)');

% ---------Experiment 3 Progagation---------

t_range = times(3,:);

% Initial conditions

index = min(find(t1 >= 4.6349));

wx3_0 = wx3(index);

wy3_0 = wy3(index);

wz3_0 = wz3(index);

w3_0 = [wx3_0, wy3_0, wz3_0]; % Vector of initial conditions for experiment 1

EOMs = @(t, omega) [ ((Iy - Iz) / Ix) * omega(2) * omega(3);

((Iz - Ix) / Iy) * omega(3) * omega(1);

((Ix - Iy) / Iz) * omega(1) * omega(2)];

% Progation using ode45

[t3_prop, omega3_prop] = ode45(EOMs, t_range, w3_0);

% Plotting

figure(6);

subplot(3,1,1);

plot(t3_prop, omega3_prop(:,1), '--r');

hold on;

title('Experiment 3 Propagated Data');

plot(t3(index:index+length(t1_prop),:), wx3(index:index+length(t1_prop),:), 'b');

hold off;

legend('Analytical', 'Experimental')

ylabel('\omega_x (rad/s)');

subplot(3,1,2);

plot(t3_prop, omega3_prop(:,3), '--r');

hold on;

plot(t3(index:index+length(t1_prop),:), wy3(index:index+length(t1_prop),:), 'b');

hold off;

legend('Analytical', 'Experimental')

ylabel('\omega_x (rad/s)');

subplot(3,1,3);

plot(t3_prop, omega3_prop(:,2), '--r');

hold on;

plot(t3(index:index+length(t1_prop),:), wz3(index:index+length(t1_prop),:), 'b');

hold off;

legend('Analytical', 'Experimental')

ylabel('\omega_x (rad/s)');

xlabel('Time (s)');

%% Final Part

% Plotting ellipsoid

% Compute semi-axes of the ellipsoid from equation above

Figure(13);

hold on;

a =

b =

c =

% Create the ellipsoid

[x, y, z] = ellipsoid(0, 0, 0, a, b, c);

surf(x, y, z, 'FaceColor', [0.5, 0.5, 0.5], 'EdgeColor',...

'y', 'FaceAlpha', 0.5); % Uniform gray